[image: image1.png]


[image: image2.jpg]


[image: image3.jpg]


                 Technical Specifications 

NEW FORD RANGER – Technical Specifications
Chassis

	Suspension
	Front – Double wishbone front suspension with coil-spring 


	
	Rear – Berlin eye leaf springs


	Steering
	Type – Electric Power Assisted Steering (EPAS), with rack and pinion gear



	
	Turning circle (m, Kerb-to-Kerb) – 11.8 (4x2); 12.4 (4x4)


	
	Turns lock-to-lock – 3.5
    

	Brakes
	Front – ventilated discs, 302 x 32 mm



	
	Rear – drums, 270 x 55 mm (4x2); 295 x 55 mm (4x4)



	
	Modulation:

Standard: Electronic Stability Control (ESC), Traction Control System (TCS), Electronic Brake Assist.

ESC system features: Hill Descent Control (HDC), Hill Start Assist (HSA), Trailer Sway Control (TSC), Load Adaptive Control (LAC), Emergency Brake Assist (EBA), Emergency Brake Light (EBL), Roll-Over Mitigation (ROM)



	Wheels & tyres
	Wheel type


Wheel size

Tyre size


	Pressed Steel / Alloy

7.0 x 16-inch

255/70 R 16
	Alloy


8.0 x 17-inch

265/65 R 17
	Alloy


8.0 x 18-inch

265/60 R 18

	
	Spare wheel:

16-inch full-size steel spare (18-inch full-size alloy wheel on Wildtrak models)



Powertrain

	Engine Type
	2.2-litre TDCi
130 PS/96 kW 
	2.2-litre TDCi
160 PS/118 kW
	3.2-litre TDCi
200 PS/147 kW

	Capacity (cc)
	2198
	2198
	3198

	Bore (mm)
	86.0
	86.0
	89.9

	Stroke (mm)
	94.6
	94.6
	100.8

	Max power (kW/PS)
	96/130
	118/160
	147/200

	At engine speed (rpm)
	3700
	3700
	3000

	Max torque (Nm)
	330 
	385
	470

	At engine speed (rpm)
	1500-2500
	1500-2500
	1500-2750

	Comp. ratio
	15.7:1
	15.7:1
	15.8:1

	Cylinders
	4 in line
	4 in line
	5 in line

	Turbocharger
	Fixed geometry turbocharger 
	Variable geometry turbocharger
	Variable geometry turbocharger

	Fuel injection
	High-pressure common-rail with fuel pressure 1800 bar
	High-pressure common-rail with fuel pressure 1800 bar
	High-pressure common-rail with fuel pressure 1800 bar

	Emission controls
	Coated Diesel Particulate Filter (cDPF)

Oxidation catalyst
	Coated Diesel Particulate Filter (cDPF)

Oxidation catalyst
	Coated Diesel Particulate Filter (cDPF)

Oxidation catalyst

	Emission level
	Water cooled EGR

Euro Stage V
	Water cooled EGR

Euro Stage V
	Water cooled EGR

Euro Stage V

	Transmission

	Transmission type: 
	Ford MT82
manual 
6-speed
	Ford MT82 manual 
6-speed
	Ford 6R80 automatic 
6-speed
	Ford MT82 manual 
6-speed
	Ford 6R80 automatic 
6-speed

	Gear ratios 
	
	
	

	6th

5th

4th

3rd

2nd

1st

Reverse

Final Drive


	0.794

1.000

1.223

1.721

2.839

5.441

4.935

3.15 (4x2)

3.15 (4x4)


	0.794
	0.691
	0.794
	0.691

	
	
	1.000
	0.867
	1.000
	0.867

	
	
	1.223
	1.143
	1.223
	1.143

	
	
	1.721
	1.521
	1.721
	1.521

	
	
	2.839
	2.342
	2.839
	2.342

	
	
	5.441
	4.171
	5.441
	4.171

	
	
	4.935
	3.400
	4.935
	3.400

	
	
	3.15 (4x2)
	-
	-
	-

	
	
	3.15 (4x4)

3.55 (4x4) optional
	3.73 (4x4)
	3.55 (4x4)
	3.73 (4x4)


	4x4 Driveline

	Type
	Driver-selectable four-wheel drive with electronically-controlled transfer box offering three modes:


Rear-wheel drive (2H)

Four-wheel drive high-range (4H)

Four-wheel drive low-range (4L)


	4x4 Transfer box
	Two speed, right hand drop, single offset, aluminium shift-on-the-fly transfer box

	4x4 ratios


	1:1 (High Range)

2.48:1 (Low Range)

	Max. speed when switching
	120 km/h


Dimensions and capacities

	Exterior (mm)
	Single Cab
	Super Cab
	Double Cab

	Overall length
	5277
	5362
	5362

	Overall width
	1860
	1860
	1860

	Overall height unladen
	1800
	1804
	1815

	Wheelbase
	3220
	3220
	3220

	Ground clearance
	229
	229
	232

	Wading depth (4x2)
	800 (600)
	800 (600)
	800

	Front track (4x2)
	1560 (1590)
	1560
	1560

	Rear track (4x2)
	1560 (1590)
	1560
	1560

	Front overhang
	905
	905
	905

	Rear overhang
	1149
	1226
	1226

	Angles (deg.)
	
	
	

	Approach angle
	28°
	28°
	28°

	Departure angle
	28°
	28°
	28°

	Ramp angle 
	25°
	25°
	25°

	Interior (mm)
	
	
	

	Front (mm)
	
	
	

	Headroom
	1024
	1024
	1022

	Max. leg room 
	1058
	1058
	1058

	Shoulder room
	1437
	1437
	1440

	Rear (mm)
	
	
	

	Headroom
	-
	938
	975

	Max. legroom
	-
	796
	902

	Shoulder room
	-
	1400
	1430

	Cargo Bed (mm)
	
	
	

	Width
	1560
	1560
	1560

	Width between arches
	1139
	1139
	1139

	Length 
	2317
	1847
	1549

	Depth
	511
	511
	511

	Loading height
	835 (724)
	835
	835


Weights

Note:  Sample data provided for selected U.K. market specification vehicles.

Please contact your local representative for details of specific models within each market.

	Versions
	Final drive ratio
	Minimum kerbweight (kg)
	Gross Vehicle Mass (kg)
	Maximum
Gross Payload (kg)
	Max. towable mass (kg)

	Single Cab
	
	
	
	
	

	2.2L TDCi 160 PS 4x4 XL
	3.15
	1931
	3200
	1269
	1800 (750)

	
	
	
	
	
	

	2.2L TDCi 160 PS 4x4 XL
	3.55
	1933
	3200
	1267
	3500 (750)

	
	
	
	
	
	

	Super Cab
	
	
	
	
	

	2.2L TDCi 160 PS 4x4 XL
	3.15
	1970
	3200
	1230
	1800 (750)

	
	
	
	
	
	

	2.2L TDCi 160 PS 4x4 XL
	3.55
	1970
	3200
	1230
	3500 (750)

	
	
	
	
	
	

	Double Cab
	
	
	
	
	

	2.2L TDCi 160 PS 4x4 Limited
	3.15
	2102
	3200
	1098
	1800 (750)

	
	
	
	
	
	

	2.2L TDCi 160 PS 4x4 automatic Limited
	3.73
	2119
	3200
	1081
	3500 (750)

	
	
	
	
	
	

	3.2L TDCi 200 PS 4x4 Limited
	3.55
	2150
	3200
	1050
	3500 (750)

	
	
	
	
	
	

	3.2L TDCi 200 PS 4x4 automatic Limited
	3.73
	2167
	3200
	1033
	3500 (750)


Minimum kerbweight represents the lightest kerbweight including full fluid levels and 90 per cent fuel levels, subject to manufacturing tolerances and options, etc. fitted. XLT, Limited and Wildtrak models will have greater kerb masses and, therefore, lower payloads due to the increased amount of standard equipment.
Towing limits quoted represent the maximum braked towing capacity of the vehicle on a 12 per cent gradient at sea level – unbraked towing capacities in brackets.

The performance and economy of all models will be reduced when used for towing. 80 kg roof load for all versions. 220 kg nose weight limit for all versions.
Economy, Performance and Emissions

Note:  Sample data provided for selected U.K. market specification vehicles.

Please contact your local representative for details of specific models within each market.

	
	Fuel consumption  l/100 km (mpg)
	PerformanceØ

	
	CO2 emissions (g/km)ØØ
	Urban
	Extra Urban
	Combined
	Max. speed km/h (mph)
	0-100 km/h
(0-62 mph) secs

	Single Cab 
	

	2.2 Duratorq TDCi (160 PS) 4x4, Final Drive 3.15
	177
	7.3 (38.7)
	6.4 (44.1)
	6.7 (42.2)
	175 (109)
	11.8

	2.2 Duratorq TDCi (160 PS) 4x4, Final Drive 3.55
	191
	8.3 (34.0)
	6.7 (42.2)
	7.3 (38.7)
	175 (109)
	11.8

	Super Cab (inc. XL, XLT and Limited)
	

	2.2 Duratorq TDCi (160 PS) 4x2, Final Drive 3.15
	177
	7.7 (36.7)
	6.3 (44.8)
	6.7 (42.2)
	175 (109)
	11.8

	2.2 Duratorq TDCi (160 PS) 4x4, Final Drive 3.15
	171
	7.1 (39.8)
	6.1 (46.3)
	6.5 (43.5)
	175 (109)
	11.8

	2.2 Duratorq TDCi (160 PS) 4x4, Final Drive 3.55
	185
	8.2 (34.4)
	6.6 (42.8)
	7.1 (39.8)
	175 (109)
	11.8

	Double Cab (inc. XL, XLT, Limited and Wildtrak)
	

	2.2 Duratorq TDCi (160 PS) 4x4, Final Drive 3.15
	171
	7.1 (39.8)
	6.1 (46.3)
	6.5 (43.5)
	175 (109)
	11.8

	2.2 Duratorq TDCi (160 PS) 4x4, Final Drive 3.55
	185
	8.2 (34.4)
	6.6 (42.8)
	7.1 (39.8)
	175 (109)
	11.8

	2.2 Duratorq TDCi (160 PS) 4x4 automatic, Final Drive 3.73
	209
	9.6 (29.4)
	7.0 (40.4)
	8.0 (35.3)
	175 (109)
	12.8

	3.2 Duratorq TDCi (200 PS) 4x4, Final Drive 3.55
	218
	9.7 (29.1)
	7.4 (38.2)
	8.3 (34.0)
	175 (109)
	10.9

	3.2 Duratorq TDCi (200 PS) 4x4 automatic, Final Drive 3.73
	234
	11.4 (24.8)
	7.4 (38.2)
	8.9 (31.7)
	175 (109)
	10.6


ØFord test figures. ØØThe declared Fuel/Energy Consumptions, CO2 emissions and electric range are measured according to the technical requirements and specifications of the European Regulations (EC) 715/2007 and (EC) 692/2008 as last amended. Fuel consumption and CO2 emissions are specified for a vehicle variant and not for a single car. The applied standard test procedure enables comparison between different vehicle types and different manufacturers. In addition to the fuel efficiency of a car, driving behaviour as well as other non-technical factors play a role in determining a car's fuel/energy consumption, CO2 emissions and electric range. CO2 is the main greenhouse gas responsible for global warming. 

Note: The data information in this press release reflects preliminary specifications and was correct at the time of going to print. However, Ford policy is one of continuous product improvement. The right is reserved to change these details at any time.

# # #

About Ford Motor Company

Ford Motor Company is a global automotive and mobility company based in Dearborn, Mich. With about 199,000 employees and 67 plants worldwide, the company’s core business includes designing, manufacturing, marketing, financing and servicing a full line of Ford cars, trucks, SUVs and electrified vehicles, as well as Lincoln luxury vehicles. At the same time, Ford is aggressively pursuing emerging opportunities through Ford Smart Mobility, the company’s plan to be a leader in connectivity, mobility, autonomous vehicles, the customer experience and data and analytics. For more information regarding Ford, its products worldwide or Ford Motor Credit Company, visit www.corporate.ford.com.
Ford of Europe is responsible for producing, selling and servicing Ford brand vehicles in 50 individual markets and employs approximately 53,000 employees at its wholly owned facilities and approximately 68,000 people when joint ventures and unconsolidated businesses are included. In addition to Ford Motor Credit Company, Ford Europe operations include Ford Customer Service Division and 24 manufacturing facilities (16 wholly owned or consolidated joint venture facilities and 8 unconsolidated joint venture facilities). The first Ford cars were shipped to Europe in 1903 – the same year Ford Motor Company was founded. European production started in 1911.
	Contact:

	Emma Bergg
					
		Ford of Europe
					
		+44 1268 405409
					
		ebergg@ford.com 

					

	


For news releases, related materials, photos and video, visit www.fordmedia.eu or www.media.ford.com. 

Follow www.twitter.com/FordEu or www.youtube.com/fordofeurope 

PAGE  
4
	For news releases, related materials, photos and video, visit www.fordmedia.eu or www.media.ford.com. 

Follow www.twitter.com/FordEu or www.youtube.com/fordofeurope 
	



